### 1. Analysis of S-Box Implementation: BRAM vs. LUTs

The S-Box lookup table was implemented using a dedicated Block RAM (BRAM) block instead of general-purpose Look-Up Tables (LUTs). This approach is superior as it conserves logic resources for other functions, provides predictable and consistent timing which simplifies design closure, and offers greater power efficiency. Furthermore, defining the S-Box contents in an external .coe file enhances design modularity, allowing for easy updates without altering the Verilog source code.

### 2. Post-Implementation Resource Utilization

The sequential FSM-based architecture proved highly efficient in resource usage. The following table summarizes the resources consumed by the design on the target Nexys-4 DDR FPGA, based on the post-placement utilization report from Vivado.

| Resource | Utilization | Available | Utilization (%) |
| --- | --- | --- | --- |
| LUTs | 75 | 63400 | 0.12% |
| Flip-Flops | 22 | 126800 | 0.02% |
| BRAM Tiles | 4 | 135 | 2.96% |
| DSP Slices | 0 | 240 | 0.00% |

### 3. Critical Path Analysis

The critical path, which limits the design's maximum frequency, is the combinational logic delay between sequential elements.

- Path Identification: The longest delay path originates from the main state_reg, passes through the NibbleSub Module (BRAM Read Access), and terminates back at the state_reg input. This path is active during the S-Box substitution states.

- Primary Delay Source: The BRAM access time is the dominant component of the delay.

- Performance Conclusion: The total path delay is well within the 10 ns clock period, confirming the design meets timing with significant positive slack.